COMMERCE SPECTRUM MANAGEMENT ADVISORY COMMITTEE (CSMAC)

SPECTRUM SHARING WORKING GROUP (SSWG)

This is a preliminary document of the sub-committee working group.  None of the material here is a final recommendation or conclusion from the sub-committee.

July 20, 2011

Working Notes

TABLE OF CONTENTS

I.	How do we set up sharing arrangements, when the primary service may continue or has the right to continue to evolve?	2

II.	What kinds of sharing are workable for industry in the long term?	2

1.	Spectrum Sharing Scenarios	2

2.	Spectrum Sharing Method Alternatives	3

a.	Spectrum Sharing Mechanism	3

b.	Method Features	3

III.	Spectrum Sharing Requirements	4

1.	Requirements of Incumbent	4

2.	Requirements of Entrant	4

IV.	Spectrum Sharing Costs	5

V.	Accommodating Changes in Incumbent Use	6

SPECTRUM SHARING WORKING GROUP WORKING NOTES:

## How do we set up sharing arrangements, when the primary service may continue or has the right to continue to evolve?

This is the first question that NTIA wants our subcommittee to answer. To answer this question we need to examine the different potential spectrum sharing scenarios (Section II) and the different requirements (Section III).  We need to determine what spectrum sharing approaches meet the requirements in the different scenarios.

## What kinds of sharing are workable for industry in the long term?

These are the potential different spectrum sharing scenarios and methods.  Some may or may not be of interest to NTIA.

### Spectrum Sharing Scenarios

### Spectrum Sharing Method Alternatives

#### Spectrum Sharing Mechanism

- Geo-Location method used to determine the transceiver parameters/capabilities (e.g., transmits frequency and power level, bandwidth, receiver capabilities).

  - Exact position vs. approximate position

  - User entered position versus GPS position versus trusted source for position

- Sensing-based method used to determine the transceiver parameters/capabilities.

  - Sensing on all entrant radios

  - Sensing on some entrant radios

  - Sensing at certain locations

  - External sensing network

  - Collaborative entrant sensing

- Combined sensing and geo-location methods used to determine the transmitted transceiver parameters/capabilities.

- Physical layer

  - Receiver ignores interference

  - Transmit modulation (UWB)

- Timesharing

  - Entrant and Incumbent share information to share spectrum in time

  - Entrant senses channel and stops transmitting rapidly when the Incumbent begins transmitting, so as not to interfere with Incumbent communication

#### Method Features

- Database connected or un-tethered method used to manage the spectrum sharing method.

  - Applied to either geographic or sensing-based methods

  - Continuous connection

  - Occasional connection (i.e. like the FCC TV whitespace Geo-Location/Database approach)

  - Periodic connection (annual)

  - Provide rule set or provide list of operating frequencies or provide operating frequency

- Interference Basis

  - Entrant/incumbent isolation determined by interference to entrant

  - Entrant/incumbent isolation determined by interference to incumbent

  - Entrant/incumbent isolation determined by interference to entrant or incumbent

## Spectrum Sharing Requirements

These are the incumbent’s and the entrant’s requirements that different spectrum sharing approaches must meet.

### Requirements of Incumbent

- Do No Harm to incumbent

- Accommodate Changes in Incumbent Use – Waveform types, occupancy, locations, etc

- Backup Band for entrant – Able reclaim the spectrum

- Enforcement – Track down interference events economically and quickly

- Safeguards/security – 	Protect against unauthorized and accidental use, avoid hackers

- DSA system diversity causing complexity – Many DSA types and entrants is too hard to manage

- Trust – Need assurance that agreement points will not change

- Security – Don’t want to reveal classified information

### Requirements of Entrant

- Do No Harm to entrant – Concerns that incumbent will have unreasonable interference criteria.  Concerns that the incumbent system receiver and other equipment characteristics are different than originally planned for.

- Safeguards/security – 	Protect against unauthorized and accidental use, avoid hackers

- Support current architecture (i.e. frequency duplex)

- Minimal changes to standards – Want to purchase standardized, non-proprietary equipment from multiple vendors

- Low prime power

- Minimal software integration costs

- Capacity – Minimal capacity lost with ‘Do No Harm’ or with fair use rules

- High reliability and assured access

- Reduce operator workload

- Trust – Need assurance that agreement points will not change

- Fair use policy

## Spectrum Sharing Costs

Table 1 shows the different spectrum sharing requirements and the approach used to meet the requirement.  Also shown are the costs for the incumbent, the entrant, and for either party to meet the requirement.

Table 1  Spectrum Sharing requirements and Costs

| Requirement | Approach Description | Cost |  |  |
| --- | --- | --- | --- | --- |
|  |  | Incumbent | Entrant | Incumbent or Entrant |
| Do No Harm | Certain frequencies at certain locations/times are unavailable for entrant use. | None | Implement dynamic network management | None |
| Do No Harm | Implement sensing-based sharing approach | Provide waveform information and equipment description. | Modify equipment to implement sensing. |  |
| Do No Harm | Implement geographic-based sharing approach | Provide and update location information and equipment description. | Modify equipment to implement position location and connection to database. | Build and operate database system. |
| Do No Harm | Implement physical layer-based sharing approach | None | Reduced link distance performance. | None |
| Do No Harm | Implement cooperative time sharing-based sharing approach | Provide and update location and schedule information and equipment description. | Modification to equipment to implement position location and connection to database. | Build and operate database system. |
| Do No Harm | Implement opportunistic time sharing-based sharing approach | None | Modification to equipment to allow rapid sensing and response to avoid interference | None |
| Accommodate Changes in Incumbent Use | Entrant equipment connected to a database. | Provide information on usage (locations, waveform types, etc). | All equipment must be periodically connected to a database. | Build and operate database system. |
| Accommodate Changes in Incumbent Use | Sensing-based approaches must have a programmable detector/classifier | Reduced flexibility in waveform design and must provide sensitive waveform information | Implement flexible, re-programmable detector/classifier. | None |
| Enforcement | Implement mechanism to detect and mitigate interference cause. | Provide information on interference event (locations, waveform types, etc). | Centralized method to locate and control equipment. | Operate interference management service. |
| Backup Band | Entrant hardware must cover multiple spectrum bands. | None | Additional hardware cost to cover additional spectrum bands. | None |
| Backup Band | Extra entrant spectrum must be provided by incumbent or entrant | Potentially need to provide additional spectrum to entrant. | Potentially need to acquire additional spectrum. | None |
| Safeguards / Security | Implement secure method to manage entrant spectrum. | None | Minimal cost, COTS solutions. | None |

## Accommodating Changes in Incumbent Use

Table 2 shows how different incumbent changes in use impact geo-location and sensing-based spectrum sharing mechanisms.  Also shown are methods that could be used to reduce this impact and to provide certainty to the entrant. Relative difficulty to implement is shown (1-easy to 3-hard).

Table 2  Incumbent Change in Use Impacts to Geo-location and Sensing-based Spectrum Sharing

| Incumbent Change in Use | Impact to Geolocation-Based Entrant Only | Impact to Sensing-Based Entrant Only | Impact to Both Entrant Types | Method to Provide Certainty to Entrant |
| --- | --- | --- | --- | --- |
| Waveform Type - modulation type, signal bandwidth or MAC (3) | None | Must have enough waveform information to design classifier(3) | None | To enable sensing approach classifier design relative to entrant waveform, incumbent provides waveform information to limit waveform parameters. |
| Mix Waveform Types Within a Band(2) | Adjust exclusion zone(1) | Implement multi-detector/classifier system(2) | None | Incumbent provides waveform types in the band |
| Withhold Transceiver Location Information | Approach not feasible(3) | None | None | Incumbent agrees to not change Transceiver Location Information policy |
| Transmit Power Level(1) | None | Change detection thresholds(1) | Decreases amount of available spectrum if sharing based on interference to entrant. | Incumbent agrees to limiting min and max transmit power level. |
| Transmit Mask Shape(1) | Adjust exclusion zone if based on entrant interference(1) | Change detection thresholds(1) | Decreases amount of available spectrum if sharing based on interference to entrant. | Incumbent agrees to limiting min and max transmit mask. |
| Desired Interference To Noise Level(1) | Adjust exclusion zone size(1) | Change detection thresholds (1) | Decreases amount of available spectrum. | Incumbent agrees to limiting interference level. |
| Number of transceivers or TX duty cycle(1) | Provide waveform information and equipment description.(1) | None | Decreases amount of available spectrum | Incumbent agrees to limiting number of TX duty cycle within each operating area. |
| Provide Entrant Advanced Warning of Transceiver Operation | Assume 100% duty cycle and reduces amount of available of spectrum, (2) | None | None | Incumbent agrees to not change advanced warning plan. |
| Receiver Selectivity(1) | Adjust exclusion zone size(1) | Change detection thresholds(1) | Decreases amount of available spectrum | Incumbent agrees to limiting adjacent channel rejection level. |
| Antenna heights or antenna gain values(1) | Adjust exclusion zone size(1) | None | Decreases amount of available spectrum |  |
| Mobility - Fixed to mobile to airborne transmitters(2) | Obtain real-time transceiver location information, use large exclusion zones, or approach not feasible(3) | None | None | Incumbent agrees to not change mobility, or to provide transceivers info in real-time to enable geolocation approach. |
| Link Type – Duplex vs telemetry vs f1/f2(2) | Adjust exclusion zone size(1) | Telemetry links require lower detection thresholds and reduces amount of available of spectrum. f1/f2 requires frequency plan information.(3) | None | Incumbent agrees to provide link type information. |

Changes to the available frequencies are potentially dynamic.  The changes may be pre-planned (i.e.24 hours notice by incumbent and then managed with a database) or may be sudden (i.e. no notice via unplanned incumbent usage change and then managed by a spectrum sensing mechanism).

Used for all spectrum sharing mechanisms (geolocation, sensing, physical layer and time sharing).